/*

* RAKSHA KAVACH – Master ESP32 Safety Firmware

* Integrates: Hardware SOS, GPS Tracking, GSM SMS Alerts, MPU6050, Pulse Sensor, and Edge AI.

*/

#include <HardwareSerial.h>

#include <TinyGPS++.h>

#include <Wire.h>

#include <Adafruit_MPU6050.h>

#include <Adafruit_Sensor.h>

// IMPORTANT: Uncomment and replace with your exact Edge Impulse library name!

// #include <Raksha_Kavach_inferencing.h>

// --- Pin Definitions ---

#define SOS_BUTTON_PIN 15

#define PULSE_SENSOR_PIN 34 // ESP32 Analog pin for Pulse Sensor

// GPS Module (Neo-6M or similar)

#define GPS_RX_PIN 16

#define GPS_TX_PIN 17

// GSM Module (SIM800L)

#define GSM_RX_PIN 4

#define GSM_TX_PIN 5

// --- Configuration ---

Const String EMERGENCY_PHONE = “112”; // Replace with your emergency contact

// --- Global Objects ---

TinyGPSPlus gps;

HardwareSerial gpsSerial(1);

HardwareSerial gsmSerial(2);

Adafruit_MPU6050 mpu;

// --- State Variables ---

Bool emergencyTriggered = false;

// --- AI Sampling Variables ---

// Note: Ensure EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE is defined by your Edge Impulse library

// If the library is not included yet, this will throw an error.

Float features[200]; // Temporary size, replace ‘200’ with EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE once library is imported

Size_t feature_ix = 0;

Unsigned long last_sample_time = 0;

Const int SAMPLE_INTERVAL_MS = 20; // 50Hz = 1 sample every 20ms

Void setup() {

Serial.begin(115200);

// Initialize SOS Button

pinMode(SOS_BUTTON_PIN, INPUT_PULLUP);

// Initialize Serial for GPS and GSM

gpsSerial.begin(9600, SERIAL_8N1, GPS_RX_PIN, GPS_TX_PIN);

gsmSerial.begin(9600, SERIAL_8N1, GSM_RX_PIN, GSM_TX_PIN);

Serial.println(“Raksha Kavach: Initializing Systems…”);

// Wake up sub-systems

initGSM();

setupSensors();

Serial.println(“Raksha Kavach: System Armed & Ready.”);

}

Void loop() {

// 1. Constantly read incoming GPS data

While (gpsSerial.available() > 0) {

Gps.encode(gpsSerial.read());

}

// 2. Check Physical SOS Button (Active Low)

If (digitalRead(SOS_BUTTON_PIN) == LOW && !emergencyTriggered) {

Serial.println(“EMERGENCY: Manual SOS Triggered!”);

triggerEmergencyProtocol();

delay(5000); // Debounce / prevent spamming

}

// 3. Run Edge Impulse AI Panic Detection

// This function manages its own 20ms timer internally

If (!emergencyTriggered) {

checkPanicAI();

}

}

// ==========================================

// SYSTEM FUNCTIONS

// ==========================================

Void initGSM() {

gsmSerial.println(“AT”);

delay(1000);

gsmSerial.println(“AT+CMGF=1”); // Set SMS to Text Mode

delay(1000);

Serial.println(“GSM Initialized.”);

}

Void setupSensors() {

pinMode(PULSE_SENSOR_PIN, INPUT);

if (!mpu.begin()) {

Serial.println(“Failed to find MPU6050 chip! Check wiring.”);

While (1) { delay(10); } // Halt system if sensor is missing

}

Mpu.setAccelerometerRange(MPU6050_RANGE_4_G);

Mpu.setGyroRange(MPU6050_RANGE_500_DEG);

Mpu.setFilterBandwidth(MPU6050_BAND_21_HZ);

Serial.println(“Sensors Initialized for AI Inference.”);

}

// ==========================================

// EDGE IMPULSE AI FUNCTIONS

// ==========================================

// Helper function for Edge Impulse to read the buffer

Int raw_feature_get_data(size_t offset, size_t length, float *out_ptr) {

Memcpy(out_ptr, features + offset, length * sizeof(float));

Return 0;

}

Void checkPanicAI() {

// 1. Maintain strict 50Hz sampling rate

If (millis() – last_sample_time < SAMPLE_INTERVAL_MS) {

Return;

}

Last_sample_time = millis();

// 2. Read Sensors

Sensors_event_t a, g, temp;

Mpu.getEvent(&a, &g, &temp);

Int pulseValue = analogRead(PULSE_SENSOR_PIN);

// 3. Fill Feature Buffer (Order must match Edge Impulse training data!)

Features[feature_ix++] = a.acceleration.x;

Features[feature_ix++] = a.acceleration.y;

Features[feature_ix++] = a.acceleration.z;

Features[feature_ix++] = pulseValue;

// 4. Run Classifier when buffer is full

// IMPORTANT: Replace ‘200’ with EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE

If (feature_ix >= 200) {

/* --- UNCOMMENT THIS BLOCK ONCE EDGE IMPULSE LIBRARY IS INSTALLED ---

Ei_impulse_result_t result = { 0 };

Signal_t features_signal;

Features_signal.total_length = sizeof(features) / sizeof(features[0]);

Features_signal.get_data = &raw_feature_get_data;

EI_IMPULSE_ERROR res = run_classifier(&features_signal, &result, false);

If (res != 0) {

Serial.printf(“ERR: Failed to run classifier (%d)\n”, res);

Feature_ix = 0;

Return;

}

For (size_t ix = 0; ix < EI_CLASSIFIER_LABEL_COUNT; ix++) {

If (String(result.classification[ix].label) == “Panic” && result.classification[ix].value > 0.85) {

Serial.println(“EMERGENCY: AI Detected Danger!”);

triggerEmergencyProtocol();

}

}

-------------------------------------------------------------------- */

Feature_ix = 0; // Reset buffer for the next window

}

}

// ==========================================

// EMERGENCY PROTOCOL FUNCTIONS

// ==========================================

Void triggerEmergencyProtocol() {

emergencyTriggered = true;

String locationLink = “Locating…”;

If (gps.location.isValid()) {

locationLink = https://maps.google.com/?q= +

String(gps.location.lat(), 6) + “,” +

String(gps.location.lng(), 6);

} else {

locationLink = “GPS signal lost. Last known area or indoor location.”;

}

String alertMessage = “ALERT! Raksha Kavach SOS triggered. Location: “ + locationLink;

Serial.println(“Executing Rescue Protocol…”);

Serial.println(alertMessage);

sendSMS(EMERGENCY_PHONE, alertMessage);

// Optional: Reset emergencyTriggered to false here if you want it to keep sending texts,

// otherwise it stays true until you reset the board.

}

Void sendSMS(String phoneNumber, String message) {

gsmSerial.println(“AT+CMGS=\”” + phoneNumber + “\””);

delay(1000);

gsmSerial.print(message);

delay(100);

gsmSerial.write(26); // ASCII code for Ctrl+Z

delay(3000);

Serial.println(“SMS Dispatched to “ + phoneNumber);

}